MOTOR FAILURE PREDICTION – STM32F401

CB.EN.U4CSE22001

CB.EN.U4CSE22011

CB.EN.U4CSE22015

CB.EN.U4CSE22036

INTRODUCTION:

This project embodies a multi-sensor system integrating ultrasonic sensors for distance measurement, a temperature sensor, and status LEDs, along with Bluetooth-based communication.Based on the readings from the sensors, an industry grade motor’s failure prone status can be identified. The key components are:

- Ultrasonic Sensors: Two sensors for distance measurements.

- Temperature Sensor: Measures temperature to control a fan.

- Status LEDs: Indicates distance thresholds for the ultrasonic sensors.

- Bluetooth Communication: Sends temperature data over UART.

- Interrupts: Configures and handles interrupts for external events.

### CODE EXPLANATION:

### Global Definitions

### 1. Power Pins

- VDD (Power Supply): Connect to +3.3V.

- VSS (Ground): Connect to GND.

### 2. Clock Pins

- HSE (High-Speed External Clock):

  - OSC_IN (PA0): Connect to the external crystal oscillator (input).

  - OSC_OUT (PA1): Connect to the external crystal oscillator (output).

- LSE (Low-Speed External Clock):

  - LSE_IN (PC14): Connect to a 32.768 kHz crystal oscillator (input).

  - LSE_OUT (PC15): Connect to the 32.768 kHz crystal oscillator (output).

### 3. Debugging and Programming Pins

- SWD (Serial Wire Debug):

  - SWDIO (PA13): Data Input/Output for debugging.

  - SWCLK (PA14): Debug clock signal.

  - NRST: Reset pin (optional for debugging).

### 4. GPIO Pins

- Configurable as Input, Output, Alternate Function (AF), or Analog.

- Refer to the STM32F401 Reference Manual for AF mappings.

Example:

- PA0: Can be used as GPIO, ADC input, or External Interrupt.

- PB6/PB7: Often used for I2C1 (SCL and SDA).

- PA9/PA10: Commonly used for USART1 (TX and RX).

### 5. Communication Interfaces

#### USART

- USART1:

  - PA9: TX (Transmit).

  - PA10: RX (Receive).

- USART2:

  - PA2: TX.

  - PA3: RX.

### 6. ADC Pins

- Analog input pins can be connected to GPIO configured as ADC:

  - PA0-PA5: ADC1 channels 0-5.

  - PB0/PB1: ADC1 channels 8/9.

### 1. Initialization

#### 1.1 System Clock Configuration

The Timer_Config function sets the system clock to use the HSI oscillator (16 MHz) and configures Timer 2 for time-based tasks, such as delay and pulse width measurement.

#### 1.2 GPIO Configuration

The GPIO_Config function initializes GPIO pins for:

- Output: Sensor triggers, LEDs, and fan.

- Input: Sensor echoes and temperature sensor.

#### 1.3 ADC Configuration

The ADC_Config function configures the ADC for analog readings from the temperature sensor.

#### 1.4 UART Configuration

The USART1_Init function sets up UART1 at 9600 baud for Bluetooth communication.

#### 1.5 NVIC Configuration

The NVIC_Config function enables and configures an external interrupt (EXTI) on PA0. This handles external events such as ultrasonic sensor measurements.

### 2. Main Function

The main function continuously:

- Measures Distance: Reads from both ultrasonic sensors using Measure_Distance1 and Measure_Distance2.

- Reads Temperature: Captures temperature data using Read_Temperature.

- Controls Fan: Adjusts the fan based on the temperature threshold using Update_Fan.

- Updates LEDs: Reflects distance thresholds for both sensors using Update_LED1 and Update_LED2.

- Sends Data: Sends temperature data over Bluetooth using printit.

### 3. Sensor Measurement and Control

#### 3.1 Distance Measurement

Measure_Distance1 and Measure_Distance2 measure the distance using the ultrasonic sensors. The process:

- Sends a 10 µs pulse via the trigger pin.

- Captures the echo pulse width using Timer 2.

- Converts pulse width to distance using the formula: Distance (cm)=Pulse Width×0.03432\text{Distance (cm)} = \frac{\text{Pulse Width} \times 0.0343}{2}Distance (cm)=2Pulse Width×0.0343​

#### 3.2 Temperature Measurement

Read_Temperature uses the ADC to read analog values from the temperature sensor and converts them to a usable format.

#### 3.3 Fan Control

The Update_Fan function turns the fan on/off based on the TEMP_THRESHOLD.

#### 3.4 LED Updates

Update_LED1 and Update_LED2 control the LEDs to indicate distance ranges:

- Red LED: Close distance.

- Yellow LED: Moderate distance.

- Green LED: Far distance.

### 4. Communication

#### 4.1 UART

USART1_Write and USART1_WriteString handle character and string transmission over UART. printit formats temperature data and sends it via Bluetooth.

#### 4.2 External Interrupt

The interrupt handler EXTI0_IRQHandler clears the pending flag for PA0 and resets LEDs tied to Sensor 2.

## Configurations and Their Uses

| Configuration | Use |
| --- | --- |
| GPIO Configuration | Initializes pins for sensors, LEDs, fan, and communication. |
| Timer Configuration | Provides accurate timing for ultrasonic sensor measurements. |
| ADC Configuration | Enables temperature sensing via analog-to-digital conversion. |
| NVIC Configuration | Enables external interrupts for sensor events. |
| UART Configuration | Facilitates Bluetooth communication for temperature monitoring. |
| PIN DIAGRAM: |  |